| M150 |  |  | Printer Sam4s Ellix (9-pins COM-port) |  |
| --- | --- | --- | --- | --- |
| Pin No. | Signal Name |  | Pin No. | Signal Name |
| 1 | GND | -> | 5 | GND |
| 8 | RXD | -> | 2 | TXD |
| 12 | TXD | -> | 3 | RXD |

One more thing: delivery set of M150 included db15 (male) – db9 cable (female), I used this one. Delivery set of Sam4S Ellix included db9 (male) – db9 port (female).

M150 cable: db15 connected to M150, i.e. we have free db9 female

Sam4s Ellix cable: db9 male connected to printer, i.e. we have free db9 female

So, just need to make a db9 male – db9 male connector using upper scheme – that’s all.