326C scale calibration
3kg

[image: image1.jpg]


[image: image9.jpg]


[image: image10.jpg]


Turn on scale,after scale will show “99999” meanwhile press “     and      ”,then press      to choice max capacity.

if scale capacity is 3kg.press     until scale show 3.000,then press     ,after scale will show “0”,press      ,scale will show “HALF”,put 1.5kg poise on plate when[image: image11.jpg]


 light is working,then press     ,after scale will show “FULL”,put 3kg poise on plate when[image: image2.jpg]


 light is working,then press      .turn off scale,it is ok for using.
6kg

Turn on scale,after scale will show “99999” meanwhile press “     and      ”,then press      to choice max capacity.

if scale capacity is 6kg.press     until scale show 6.000,then press     ,after scale will show “0”,press      ,scale will show “HALF”,put 3kg poise on plate when[image: image3.jpg]


 light is working,then press     ,after scale will show “FULL”,put 6kg poise on plate when[image: image4.jpg]


 light is working,then press      .turn off scale,it is ok for using.

15kg

Turn on scale,after scale will show “99999” meanwhile press “     and      ”,then press      to choice max capacity.

if scale capacity is 15kg.press     until scale show 15.000,then press     ,after scale will show “0”,press      ,scale will show “HALF”,put 7.5kg poise on plate when[image: image5.jpg]


 light is working,then press     ,after scale will show “FULL”,put 15kg poise on plate when[image: image6.jpg]


 light is working,then press      .turn off scale,it is ok for using.

30kg

Turn on scale,after scale will show “99999” meanwhile press “     and      ”,then press      to choice max capacity.

if scale capacity is 30kg.press     until scale show 30.000,then press     ,after scale will show “0”,press      ,scale will show “HALF”,put 15kg poise on plate when[image: image7.jpg]


 light is working,then press     ,after scale will show “FULL”,put 30kg poise on plate when[image: image8.jpg]


 light is working,then press      .turn off scale,it is ok for using.

