CIS Image Factory_U
User Guide

( version 3.4.2 )
[image: image1.jpg]See:IT‘ech

SEETECH CO., LTD.




※ CIS Image Factory Download -> Click Here
How to use the program with machine
(1) Connect the machine to PC with serial cable (15pin-9pin)

(2) Turn the power on.

(3) The machine show below message on the LCD DISPLAY.

 SHAPE  \* MERGEFORMAT 



Any Operation Mode is available to calibrate the machine except standard counting Mode.
Select currency to USD.
(4) Press and hold down the RESTART Key. The LCD DISPLAY shows as below.

 SHAPE  \* MERGEFORMAT 



(5) Enter the PIN CODE, 

Press and hold down +10 → CURRENCY → Press and hold down RESTART 

→ CF → CURRENCY → MODE → PRINT”.

(6) Display shows as below.

 SHAPE  \* MERGEFORMAT 



(7) To exit the TECHNICAL SERVICE MENU, press the RESTART key.

Press and hold down the CURRENCY key and enter CIS TEST MODE.
(8) Select 6. CIS INFORMATION FLAG * ALL.
[image: image5.jpg]EMATION FLAG HEE
HLE

1





(9) By pressing +1 Key, select 6. CIS INFORMATION FLAG * E107
[image: image6.png]TLPRESS RESTHET KEH
SOFTWARE VERSIOH
SETTLHGE HMEBEE
EMHCODER SETTIHG HOBE
BORRD STRTUS MEBE

INFORMATION FLAG





(10)  Exit CIS TEST MODE.
(11)  Execute CIS Image Factory_U_3.4.2.

[image: image7.jpg]< CI8 Image Factory For User  ver 3,37  (2008,04,14)
File Edit View Help

(DS |4 =28 ® |5 P[kv|vs o okn

Port Baud Rate Data Bits Stop Bits Parity Bits
comt =|[11sem =lfs <|[1|[None =] @] CONNECT RESET | REPORT|

Full Image Partl Image Part? Image Part3 Image Partd Image Part5 Image Free Image.
o xfo o xfo o xJo o xfo o xfo o xo [400 x [1a0 TEXT|
I T T T

Line: 0 Col: 0




(12)  Set COM Port.
After select COM Port which is connected with machine by serial cable, set below parameters.
Baud Rate: 115200

Data Bits: 8

Stop Bits: 1

Parity Bits: None

(13)  Click CONNECT button. If machine connects with PC properly, red light turns green.

[image: image8.jpg]Port Baud Rate Data Bits Stop Bits Parity Bits —
[comt ~[[115200 ~[[6 ~|[i ~|[Nene ~| L.} CO"'§CT 1




 

[Before Connecting]
[image: image9.jpg]Port

Baud Rate Data Bits Stop Bits Parity Bits

||

||

Bl

||

=

© | | DISCONNECT|

i





[After Connecting]
(14)  Put banknotes which make E107 on the Hopper.
(15)  When E107 error is caused, the machine sent to data to PC as below.
[image: image10.png]FREE IMAGE MODE - 100000aw, - =3

REPORT LOAD
x v ReB: Magn: x1

X= 1600, = 400

TEXT Window





(16)  Click SAVE button to save image data.
(17)  The error data must be saved from 10 notes at least.

 Mix     RJP      CF


--------------------


        


    0        0


    


 $          0


    


--------------------


 B100 S1000      ---





  TECHNICAL SERVICE MENU


------------------------------











  PIN CODE : -------











------------------------------


            EXIT – RESTART KEY











  TECHNICAL SERVICE MENU


------------------------------








  PIN CODE : *******








  PIN CODE approved !!





------------------------------


            EXIT – RESTART KEY





E107


E107





Click “SAVE” for saving data!








