If you change or upgrade board(Main, CF, CIS...), you must set board

instruction as below.

9-3. SETTING INSTRUCTION

9-3-1. MAIN BOARD SETTING

(1) Turn the power on.

(2) The machine show below message on the LCD DISPLAY.
[image: image1.png]B100 $1000





Any Operation Mode is available to calibrate the machine except standard counting

Mode.

(3) Press and hold down the RESTART Key. The LCD DISPLAY shows as below.
[image: image2.png]TECHNICAL SERVICE MENU

PIN CODE

EXIT - RESTART KEY





(4) Enter the PIN CODE,

Press and hold down +10 →CURRENCY →Press and hold down RESTART

→CF →CURRENCY →MODE →PRINT”.

(5) Display shows as below.
[image: image3.png]TECHNICAL SERVICE MENU

PIN CODE : sxkxkx

PIN CODE approved !!

EXIT - RESTART





(6) To exit the TECHNICAL SERVICE MENU, press the RESTART key.
(7) Press and hold down the RESTART Key until the machine show below

message on the LCD DISPLAY.
[image: image4.png]SELECT SETTING SENSOR GROUP.

EXIT ( RESTART KEY )
CMAIN PART

. GF PART

.CIS PART

.0.C. REPORT





(8) Press the MODE Key until the machine show below message on the LCD

DISPLAY.
[image: image5.png]SELECT SETTING SENSOR GROUP

EXIT ( RESTART KEY )
CMAIN PART

CF PART

.CIS PART

.0.C. REPORT





The item No. 2 is selected.

(9) Press the MODE key and select the item 2, 2. MAIN PART.

(10) Press the PRINT key and enter the item 2, 2. MAIN PART.

(11) Display shows as below.
[image: image6.png]SELECT SETTING SENSOR GROUP

351 EXIT(PRESS RESTART KEY)
2.H,5 R, B SENSOR
3.COUNT SENSORS
4.MOTOR SPEED CALIBRATING





(12) Press MODE key and select the item3, 3. COUNT SENSORS.

(13) Press PRINT key and enter the item 3, 3. COUNT SENSORS.

(14) Display shows as below.
[image: image7.png]COUNT SENSORS SETTING MNODE

SOEXITCC  SAVE LOAD

Sen-| Offsets |Set Value|Set-
Serel LR L R

Cis 14095, 4095|x 40,
Main|4095, 4095|x 30, 30x|
14095, 4095]x 20, 20x]|





(15) Press MODE key and select CIS sensor.

(16) Select the item as below.
[image: image8.png]COUNT SENSORS SETTING MODE
SAVE LOAD

Sen-| 0ffsets |Set Value|Set-
Supfbe R Ry

Cis |4095,4095|x 40, 40x| X <<
Main|4095, 4095|x 30, 30x| X
Rej. 4095, 4095[x 20, 20x]





(17) Press PRINT key and set the count sensor it places front side of CIS sensor.

After setting, display shows as below. It is possible the real setting value is

different.
[image: image9.png]COUNT SENSORS SETTING MODE
EXIT SAVE LOAD

Sen-| Offsets [Set Value|Set-
Sorsl L R | ting
- -+

Cis |lﬂ85 4095|x 53, 35x| 0 <<
Main|4095, 4095]x 30, 30x| X
Rej. 4095, 4095|x 20, 20x| X





(18) Press MODE key and select Rej. Sensor. Display shows as below.
[image: image10.png]COUNT SENSORS SETTING NODE

EXIT SAVE LOAD

Sen-| Offsets |Set Value|Set-
Sorsl LoRho bR b ting

Cis |4095,4095|x 53, 35x| 0
Main|4095, 4095|x 30, 30x| X
Rej. Ilues 4095|x 20, 20x| X <<





(19) Press PRINT key and set the count sensor it places the front side of the

REJECT sensor. Display shows as below. It is possible the real setting value is

different.
[image: image11.png]COUNT SENSORS SETTING MODE
EXIT SAVE LOAD

Sen-| 0ffsets |Set Value|Set-
Sarad LR R

Cis |4095,4005|x 53, 35x|
Main|4095, 4095|x 30, 30x|
Re Itnns 4095[x 16, 14x|





(20) Press MODE key and select Main. Sensor. Display shows as below.
[image: image12.png]COUNT SENSORS SETTING MNODE

SAVE LOAD

Sen-| Offssts |Set Valus|Set-
Sors| L R | ting

+

Gis 14095 40951x 53 35x] ©

M 14095, 4095|x 30, 30x| X <<
14095, 4095[x 16, 14x| 0





(21) Insert the calibration card for count sensor to the machine.

Calibration Cards for Count Sensor.
[image: image13.png]70mm

140mm




[image: image14.png]



Put the calibration card on HOPPER.
[image: image15.png]



Turn the KICKER ROLLER and insert the card until appear the display below.
Display shows as below.
[image: image16.png]COUNT SENSORS SETTING NODE
SAVE

Sen-| |set Value|Set-
Sors| {1538 aanen TERSIRE 401} ¢

Cis x 53, 3sx(fo
Main|4095, 409580 30, 300lf X <<

Rej





[image: image17.png]Cis |4095,
Main|4095,
Rej. 4095,

Cis |4095,
Main|4095,
Rej. 4095,

409
409
409

409
409
409





Insert the paper until the value change (o -> x )
(22) Press PRINT key and calibrate MAIN count sensor. After calibrating, display

shows as below. It is possible the real setting value is different.
[image: image18.png]COUNT SENSORS SETTING NODE

SAVE LOAD

Sen-| Offsets |Set Value|Set-

Sors| L | ting
Esgiil —mpme sl
Cis |4095,4005|x 53, 35x| 0
Nain|4095,4095]0 32, 450] 0 <<
Rej. 4095, 4095|x 16, 14x| 0





(23) Remove the calibration card for count sensor from the machine turning the

roller.

(24) Press MODE Key and select save menu.
[image: image19.png]COUNT SENSORS SETTING MODE

EXIT  >>SAVECC  LOAD

Sen-| Offsets |Set Value|Set-
e L

Cis ]4095,4005]x 53, 35x| 0
Main|4095,4095[x 30, 30x| 0 <<
0





(25) Press PRINT key and save the changed data.

(26) Press RESTART key and exit COUNT SENSORS SETTING MODE.
[image: image20.png]SELECT SETTING SENSOR GROUP

1.EXIT(PRESS RESTART KEY)
2.H.S.R.B SENSOR
>>3.COUNT SENSORS

4.MOTOR SPEED CALIBRATING





(27) Press PRINT key and enter item4, 4. MOTOR SPEED CALIBRATING. Display

shows as below.
[image: image21.png]



(28) Press ADD key to find speed automatically. It takes about 2 minutes.

(29) During measuring the speed, display shows as below.
[image: image22.png]SPEED SETTING

oW





[image: image23.png]MOTOR SPEED SETTING

TS/M

600





(30) After measuring, display shows as below.
[image: image24.png]MOTOR SPEED SETTING WODE

EXIT RESTART

5
SAVE v, pwn > PRINT
>

SAVE v only cF

TEST | xxxx | xxxx





(31) Press PRINT key and save the data, MOTOR PWM value and CHECK

VALUE.
[image: image25.png]SPEED SETTING





(32) Press RESTART key and finish the calibration for MAIN board.

(33) Turn off the machine.

(34) Tune on the machine again.
9-3-2. CF BOARD SETTING
CAUTION!
Check CF board software version of the technical service menu

before cf board setting.
1. UV SETTING

(1) Turn the power on.

(2) The machine show below message on the LCD DISPLAY.
[image: image26.png]



Any Currency Mode is available to calibrate the machine.

(3) Press and hold down the RESTART Key. The machine show below message on the LCD

DISPLAY.
[image: image27.png]TECHNICAL SERVICE MENU

EXIT - RESTART KEY





(4) Enter the PIN CODE,

Press and hold down +10 →CURRENCY →Press and hold down RESTART

→CF →CURRENCY →MODE →PRINT”.

(5) Display shows as below.
[image: image28.png]TECHNICAL SERVICE MENU

PIN GODE : sxxrakx

PIN CODE approved Il

EXIT - RESTART





(6) If you want to exit the TECHNICAL SERVICE MENU, press the RESTART key.

(7) Press and hold down the RESTART Key until the machine show below message on the

LCD DISPLAY.
[image: image29.png]SELECT SETTING SENSOR GROUP

>>1.EXIT ( RESTART KEY )
2. MAIN PART
3.CF PART
4.C1S PART

5.0.C. REPORT





(8) Press the MODE Key until the machine show below message on the LCD DISPLAY.
[image: image30.png]SELECT SETTING SENSOR GROUP

TEXIT ( RESTART KEY )
MAIN PART
LGF PART
CIS PART
5.0.C. REPORT





The item No. 3 is selected.

(14) Press the MODE key and select the item 3, 3. CF PART.

(15) Press the PRINT key and enter the item 3, 3. CF PART.
[image: image31.png]CF -> UV, Ma

CURRENCY > IR





Press the CF key and display shows as below.
[image: image32.png]SELECT SETTING CURRENCY

- EUROPEAN
~AMERICA
CSYRIA

~SOUTH AFRICA
. TURKEY
- ENGLAND





(16) Press the MODE key and select the item 1, 1. EUROPEAN.
(17) Press the PRINT key and enter the item 1, 1. EUROPEAN.
[image: image33.png]CF GROUP OF EUROPEAN

2.M8 NODE

>>3.DEFAULT DATA LOAD
4. FACTORY SETTING LOAD
5. SERIAL TRANS





(18) Press the MODE key and select the item 3, 3.DEFAULT DATA LOAD.

(19) Press the PRINT key and select the item 3, 3.DEFAULT DATA LOAD.

(20) Press the MODE key and select the item 1, 1.UV MODE

(21) Press the PRINT key and select the item 1, 1.UV MODE
[image: image34.png]_1.UV OFFSET SETTING**
EUR BL : 39% BR : 30%
OFF REF EXIT

DE) [R(PR
236] 146
1461 131
132] 129
217] 128

<<Calibration offset>>





(22) Press the PRINT key for CBR setting.
[image: image35.png]#%1_1. UV OFFSET SETTING##
C : EUR BL : 39% BR : 39%
>>CBR OFF REF EXIT

L(PR DE) [R (PR DE)
143 236] 146 236
136 146] 131 153




The value will be change.
It is possible the real setting value is different.

(23) Press the MODE key and select the OFF mode.
[image: image36.png]#x1_1.UV OFFSET SETTING#x
C : EUR BL : 39% BR : 39%
CBR >>0FF REF EXIT

DE) |R (PR

236 146
1461 131
1321 129
2191 126

<<Calibration offset>>





(24) Prepare the Black setting paper as below.
[image: image37.png]140mm

—>





(25) Put the Black setting paper on Hopper and press the PRINT key.
[image: image38.png]



(26)The Black setting paper feeds and starts calibration.
[image: image39.png]



Display shows as below.
[image: image40.png]#x1_1.UV OFFSET SETTINGsx

¢ : EUR BL : 39% BR : 39%
CBR >>0FF EXIT

7
41| 132 144
4] 129 147

<<Calibration offset>>




The value will be change.
After finishing calibration, the Black paper out the Main Pocket.

(27)Press the CURRENCY key and enter the UV GAIN SETTING mode.
[image: image41.png]#%1_2.UV GAIN SETTING**
C: EUR BL : 39% BR : 39%
S>TUNE  DETAIL REF EXIT

CHI

<Clnsert Setting Paper>>





(28) Prepare the Blue setting paper as below.(10 sheets)

Be careful!! This is not same the paper of CF series.

Please use it separates certainly.
[image: image42.png]70mm

140mm




(29)Press the PRINT key and enter the UV GAIN DETAIL MODE.
[image: image43.png]#%1_2. UV GAIN DETAIL WODEx#
¢ : EUR PN : 10
>>TUNE
RMax MEASURE RMin
US)L: 36% > XXX% 34%
R: 36% > XXX% 34y
BS)L: 36% > XXX% 348

FS)L: 26% > XXX% 24%
R: 26% > XXX 24%
<<UV GAIN DETAIL ADJUST>>

>
3
>
R: 36% > XXX% >  34%
>
>





(30)Put the Blue setting paper(10 sheets) on Hopper and press the PRINT key.
[image: image44.png]



If the PN is not correct or MEASURE value is not be between RMax and RMin, display

show as below.
[image: image45.png]#%1_2.UV GAIN DETAIL MODEx»
C : EUR PN : 10
>>TUNE
RMax MEASURE RMin
US)L: 36% > XXX% 34%
R: 36% > XXX 34%

R: 36% > XXX§ 34%
FS)L: 26% > XXX% 24%

<REPEAT DETAIL TUNE>)]

>
>

BS)L: 36% > XXX% >  34%
>
>





(31)After finishing the calibration, display as below.
[image: image46.png]#%1_2.UV GAIN DETAIL WODE*+
¢ : EUR PN : 10
>>TUNE EXIT

<<UV SETTING COMPLETE!1>>





[image: image47.png]#%1_2.UV GAIN SETTING*»
C : EUR BL : 39% BR
>>TUNE  DETAIL REF




The value will be change.
(32)Press the RESTART key and exit UV GAIN SETTING mode.
[image: image48.png]CF GROUP OF EUROPEAN
DOEXIT

UV NODE

WG NODE

.DEFAULT DATA LOAD

. FACTORY SETTING LOAD
_SERIAL TRANS





(33) Press the RESTART key and exit mode.

Display shows as below.
[image: image49.png]SELECT SETTING CURRENCY

. EUROPEAN

L AMERICA
LSYRIA

. SOUTH AFRICA
. TURKEY
LENGLAND





(34) Press the MODE key and select the item 2, 2. AMERICA

(35) Press the PRINT key and enter the item 2, 2. AMERICA

(36) Repeat the (18)~(31).

(37) Press the RESTART key and exit UV GAIN SETTING mode.
[image: image50.png]CF GROUP OF AMERICA

SYEXIT
UV WODE
MG NODE
_DEFAULT DATA LOAD
_FACTORY SETTING LOAD
SERIAL TRANS





(38) Press the RESTART key and exit mode.

Display shows as below.
[image: image51.png]SELECT SETTING CURRENCY

1

EUROPEAN
AMERICA
SYRIA

LSOUTH AFRICA

TURKEY

LENGLAND





(39) Press the MODE key and select the item 3~6.

(40) Press the PRINT key and enter the item 3~6.

(41) Repeat the (18)~(31).

(42) Press the RESTART key and exit UV GAIN SETTING mode.
[image: image52.png]CF GROUP OF LOCAL CURRENCY

YOEXIT
UV NODE
MG WODE
DEFAULT DATA LOAD
_FACTORY SETTING LOAD
SERIAL TRANS





(43) Press the RESTART key and exit mode.

If all currencies finished calibration, press the RESTART key and exit mode.
[image: image53.png]



2. CHECKING MG SENSOR
(1) Turn the power on.

(2) The machine show below message on the LCD DISPLAY.
[image: image54.png]



Any Currency Mode is available to calibrate the machine.

(3) Press and hold down the RESTART Key. The machine show below message on the LCD

DISPLAY.
[image: image55.png]TECHNICAL SERVICE MENU

EXIT - RESTART KEY





(4) Enter the PIN CODE,

Press and hold down +10 →CURRENCY →Press and hold down RESTART

→CF →CURRENCY →MODE →PRINT”.

(5) Display shows as below.
[image: image56.png]TECHNICAL SERVICE MENU

PIN GODE : sxxrakx

PIN CODE approved Il

EXIT - RESTART





(6) If you want to exit the TECHNICAL SERVICE MENU, press the RESTART key.

(7) Press and hold down the RESTART Key until the machine show below message on the

LCD DISPLAY.
[image: image57.png]SELECT SETTING SENSOR GROUP

>>1.EXIT ( RESTART KEY )
2. MAIN PART
3.CF PART
4.C1S PART

5.0.C. REPORT





(8) Press the MODE Key until the machine show below message on the LCD DISPLAY.
[image: image58.png]SELECT SETTING SENSOR GROUP

TEXIT ( RESTART KEY )
MAIN PART
LGF PART
CIS PART
5.0.C. REPORT





The item No. 3 is selected.

(14) Press the MODE key and select the item 3, 3. CF PART.

(15) Press the PRINT key and enter the item 3, 3. CF PART.
[image: image59.png]CF -> UV, Ma

CURRENCY > IR





Press the CF key and display shows as below.
[image: image60.png]SELECT SETTING CURRENCY

- EUROPEAN
~AMERICA
CSYRIA

~SOUTH AFRICA
. TURKEY
- ENGLAND





(15) Press the MODE key and select the item 1, 1. EUROPEAN
(16) Press the PRINT key and enter the item 1, 1. EUROPEAN

Display shows as below.
[image: image61.png]CF GROUP OF EUROPEAN

SYEXIT
UV NODE
MG _NODE
.DEFAULT DATA LOAD

.FACTORY SETTING LOAD
SERIAL TRANS





(17) Press the MODE Key until the machine show below message on the LCD DISPLAY.
[image: image62.png]CF GROUP OF EUROPEAN

1.UV MODE
>>2. WG WODE
3.DEFAULT DATA LOAD

4.FACTORY SETTING LOAD
5. SERIAL TRANS





(18) Press the MODE key and select the item 2, 2.MG MODE.

(19) Press the PRINT key and select the item 2, 2.MG MODE.
[image: image63.png]#x 2.MG TEST MODE *»
C:EUR D: & Vers:0LD
>>TEST THREAD EXIT

CHI CH2 CH3 CH4 CH5 CHE
MINZXXX XXX XXX XXX XXX XXX

MAX:XXX XXX XXX XXX XXX XXX

THREAD: 20
TOTAL : XXX-XXX
<< UV TEST 5>





(20) Put the USD $1 or $2 on Hopper and press the PRINT key.
[image: image64.png]#% 2. MG TEST MODE *=
CIEUR D: § Vers:0LD
>>TEST THREAD EXIT

CHI CH2 CH3 CH4 CH5 CH6
Minf23 28 40 36 28 21
Maxf23 28 40 36 28 21

THREAD: 20
TOTAL : 176-176
KGN iTESTH)




The value will be change.
9-3-3. IR BOARD SETTING
(1) Turn the power on.

(2) The machine show below message on the LCD DISPLAY.
[image: image65.png]



Any Currency Mode is available to calibrate the machine.

(3) Press and hold down the RESTART Key. The machine show below message on the LCD

DISPLAY.
[image: image66.png]TECHNICAL SERVICE MENU

EXIT - RESTART KEY





(4) Enter the PIN CODE,

Press and hold down +10 →CURRENCY →Press and hold down RESTART

→CF →CURRENCY →MODE →PRINT”.

(5) Display shows as below.
[image: image67.png]TECHNICAL SERVICE MENU

PIN GODE : sxxrakx

PIN CODE approved Il

EXIT - RESTART





(6) If you want to exit the TECHNICAL SERVICE MENU, press the RESTART key.

(7) Press and hold down the RESTART Key until the machine show below message on the

LCD DISPLAY.
[image: image68.png]SELECT SETTING SENSOR GROUP

>>1.EXIT ( RESTART KEY )
2. MAIN PART
3.CF PART
4.C1S PART

5.0.C. REPORT





(8) Press the MODE Key until the machine show below message on the LCD DISPLAY.
[image: image69.png]SELECT SETTING SENSOR GROUP

TEXIT ( RESTART KEY )
MAIN PART
LGF PART
CIS PART
5.0.C. REPORT





The item No. 3 is selected.

(14) Press the MODE key and select the item 3, 3. CF PART.

(15) Press the PRINT key and enter the item 3, 3. CF PART.
[image: image70.png]CF -> UV, Ma

CURRENCY > IR





(16) Press the CURRENCY key and display shows as below.
[image: image71.png]IR SETTING SELECT

>>1. IR OFFSET SETTING
2. 1R GAIN SETTING
3. IR VIEW IMAGE





(17) Press the MODE key and select the item 1. IR OFFSET SETTING.

(18) Press the PRINT key and enter the item 1. IR OFFSET SETTING.
[image: image72.png]255
255
255

255
255
255
255
255

DIR:F

CHY:
CH10:
CHIT:
CH12:
CH13:
CHI4:
CH15:
CH16:

255
255
255
255
255
255
255
255

#1_1.IR OFFSET SETTING *x

EXIT
255
255
255
255
255
255
255
255





(19) Prepare the Black setting paper as below.
It is differ from CF setting paper.

[image: image73.png]


 Insertion direction
(20) Put the Black setting paper on Hopper and press the PRINT key.
[image: image74.png]



(21) The Black setting paper feeds and starts calibration.
[image: image75.png]



Display shows as below.
[image: image76.png]#x1_1. IR OFFSET SETTING %

>>0f

CHI:

FF

0
0
10
0
0

CHY:

DIR:F

CHIT:

E
CH25:

#%1_1.1R OFFSET SETTING

>>0FF

CH1
cH2
CH3
CH4
CHS
CHE
cHT
CHB

1101
1103
1103
1101
1103
1103
1102
1103

97
83
125
103
122
103
90
134

DIR:F

cHY
cH10
CHTT
cHi2
CHT3
cHia
CH15
CH16

1103
$103
1100
1100
1103
1103
101
1103

T




Progress direction
If the IR channel has a problem, “X” mark is shown.
[image: image77.png]CH1:
CH2:
CH3:
CH4:
CH5:
CHE:
CHT:
CHB:

CHISG:
CH16:

X
X
X
0
™0

0

0

CH17:
CH18:
CH19:
CH20:
CH21:
CH22:
CH23:
CH24:

Scocococoso

CH25:
CH26:
CH27:
CH28:
CH29:
CH30:
CH31:
CH32:




(22) Press RESTART Key or select the EXIT using MODE Key and exit IR OFFSET

SETTING Mode.

(23) Press the MODE key and select the item 2. IR GAIN SETTING.

(24) Press the PRINT key and enter the item 2. IR GAIN SETTING.

- IR GAIN SETTING
[image: image78.png]#x1_2. IR GAIN SETTING **
DIR:F TYPE:AL  PAPER:10
S>GAIN DETAIL EXIT
CH1:128 CH9:128
CH2:128 CH10:128
CH3:128 CH11:128

CH4:128 CH12:128
CH5:128 CH13:128
CH6:128 CH14:128
CH7:128 CH15:128
CH8:128 CH16:128





(25) Put the Blue setting paper on Hopper and press the PRINT key
(26) The Blue setting paper feeds and starts calibration.

[image: image79.png]140mm

175mm




Insertion direction
[image: image80.png]



Display shows as below
[image: image81.png]#%1_2. IR GAIN SETTING *x
DIR:F TYPE:RL  PAPER:10
>>GAIN DETAIL EXIT
CH1:0 CH9:0 CH1 CH25:0




Progress direction
If the IR channel has a problem, “X” mark is shown.

(27) Prepare the Blue setting paper as below. (10 sheets)
[image: image82.png]70mm

176mm





(28) Press the MODE key and go to DETAIL and put the Blue setting paper (10 sheets) on

Hopper and press the PRINT key.
[image: image83.png]



- GAIN DETAIL SETTING
[image: image84.png]#%1_2. IR GAIN SETTING #=
DIR:F TYPE:RL PAPER:10
GAIN >>DETAIL EXIT

Setting Completel!

CHB:0 CH16:0 CH24:0 CH32:0





When the setting is not complete, following message shows.

Almost completed one or two steps, but if not completed more than 5 steps then check

IR sensor.
[image: image85.png]#%1_2 IR GAIN SETTING *#
DIR:F TYPE:RL PAPER:10
GAIN >>DETAIL EXIT

Retry Setting

CHB:0 CH16:0 CH24:0 CH32:0





If inserted notes are different from displayed number, following message shows.
[image: image86.png]*%1_2. IR GAIN SETTING *x
DIR:F TYPE:RL
GAIN >>DETAIL

heck Paper Number

CHB:0 CH16:0 CH24:0 CH32:0





9-3-4. CIS BOARD SETTING

(1) Turn the power on.

(2) The machine show below message on the LCD DISPLAY.
[image: image87.png]



Any Currency Mode is available to calibrate the machine.

(3) Press and hold down the RESTART Key. The machine show below message on the LCD

DISPLAY.
[image: image88.png]TECHNICAL SERVICE MENU

EXIT - RESTART KEY





(4) Enter the PIN CODE,

Press and hold down +10 →CURRENCY →Press and hold down RESTART

→CF →CURRENCY →MODE →PRINT”.

(5) Display shows as below.
[image: image89.png]TECHNICAL SERVICE MENU

PIN GODE : sxxrakx

PIN CODE approved Il

EXIT - RESTART





(6) If you want to exit the TECHNICAL SERVICE MENU, press the RESTART key.

(7) Press and hold down the RESTART key and enter SELECT SETTING SENSOR GROUP.
[image: image90.png]SELECT SETTING SENSOR GROUP

1.EXIT ( RESTART KEY )
2. MAIN PART
3.CF PART

>>4.C1S PART

5.0.C. REPORT





The item No. 4 is selected.

(8) Press the MODE key and select the item 4. CIS PART

(9) Press the PRINT key and enter the item 4. CIS PART

(10) Display shows as below.
[image: image91.png]>> [1] White Paper Setting
[2] Black Paper Setting
[3] Light Value : [052]
4] EXIT





(11) Select the item White Paper.

(12) Prepare the jig for CIS sensor.
[image: image92.png]



Calibration paper
(13) Clean the CIS sensor using soft colth.

(14) Put the jig to CIS sensor as following procedure.
[image: image93.png]



Open Rear Cover.
[image: image94.png]



Prepare CIS jig.
[image: image95.png]



[image: image96.png]A




After set the calibration paper

opposite CIS sensor part and put the

jig on the left side of CIS sensor

referring to picture.
[image: image97.png]



Adjust the other side of jig to CIS sensor.
[image: image98.png]



Pull the metal plate like a picture and fix the jig to detector module.
[image: image99.png]



[image: image100.png]



The metal plate holds the detector module.
[image: image101.png]



Cautions!!

Before put the jig, check the calibration paper side which is scratched or not.

If the paper parts is damaged, the calibration does not proper.

In this case, please replace the paper with new one.

We support the Gray paper type with calibration cards which is tape type.
(15) Adhere the calibration card to the CIS sensor and attach it using tape or etc.

(16) Press the PRINT key to calibrate white paper.
[image: image102.png]5> [1] White Paper Setting
[2] Black Paper Setting
[3] Light Value : [052]
[4] EXIT




After 2~3 seconds, the value will be changed. The value should be over 100.
It is possible the real setting value is different.

(17) Remove the jig from CIS sensor.

(18) Close the rear cover.
(19) Press the MODE key and select the item Black Paper.
[image: image103.png][1] White Paper Setting
>> [2] Black Paper Setting

[3] Light Value

[4] EXIT





(20) Press the PRINT key.

(21) Display shows as below.
[image: image104.png][1]1 White Paper Setting
>> [2] Black Paper Setting
[3] Light Value :
[41 EXIT

W] Ave
(W] Min




The value will be changed.

The value should be under 30.
It is possible the real setting value is different.

(22) Press the RESTART key to save data and exit CIS SETTING MODE.
