3. Coin outlet plate adjust
Notes: the height between coin outlet plate and rail is to ensure the correct sorting of different coins, so it is important to adjust it well. 

Adjustment method as below： 

(1) [image: image1.jpg]


Loose the M3 bolt in picture 8-2 to adjust the height between coin outlet plate and rail. When you adjust the first coin outlet plate(from right to left), you need to use a coin of current channel and next 
Picture 8-2
channel. Please use a coin of next channel to stick onto the side plate and bottom rail, adjust the coin outlet plate, to make its  vertical distance with next channel’s coin is 0.3-05mm,then use the coin of current channel to stick onto the side board, push the coin to the adjusted coin outlet plate, the coin should be at least 0.3mm higher than the bottom of the coin outlet plate and use torsion 5kg/cm to fix the coin outlet plate. You can use the same way to adjust other coin outlet plates. 

At last take out the coin with smallest diameter, stick the coin onto the rail, make sure the bottom of the coin outlet plate is 1mm lower than the top of the coin, then tighten the bolt.                          

